Ralink WPS Capable AP

## WPS Commands Descriptions

- Disable WPS function support

  - iwpriv ra0 set WscConfMode=0

- Enable WPS function support

  - iwpriv ra0 set WscConfMode =7 (Binary: 111)

(AP could be Registrar(0x4), Proxy(0x2) or Enrollee(0x1))

- WPS AP SC (Simple Config) State

  - iwpriv ra0 set WscConfStatus=1 (AP is un-configured)

  - iwpriv ra0 set WscConfStatus=2 (AP is configured)

- WPS Configured Methods

  - iwpriv ra0 set WscMode =1 (use PIN code)

  - iwpriv ra0 set WscMode =2 (use PBC)

- Input Enrollee’s Pin Code to AP-Registrar

  - iwpriv ra0 set WscPinCode=xxxxxxxx

- Reset WPS AP to the OOB configuration

  - iwpriv ra0 set WscOOB=1

(Security: WPAPSK/TKIP, psk: “RalinkInitialAPxx1234” ; SC state: 0x1)

(SSID: RalinkInitialAPxxxxxx, last three characters of AP MAC address)

- Trigger WPS AP to do simple config with WPS Client

  - iwpriv ra0 set WscGetConf=1

- WPS AP doesn’t need those commands above(except “iwpriv ra0 set WscConfMode =7”) when WPS AP services as proxy

## How to know WPS AP services as Internal Registrar, Enrollee or Proxy

- It depends on the content of EAP-Response/Identity from WPS Client.

  - When identity is “WFA-SimpleConfig-Registrar-1-0”:

WPS AP would service as Enrollee. (After set trigger command)

  - When identity is “WFA-SimpleConfig-Enrollee-1-0”:

WPS AP would service as Internal Registrar and Proxy.

Without trigger command, WPS AP services as proxy only.

## Ralink WPS AP could service as external Registrar by UpnP

## Example

- Reset to un-configured out-of-box setting

  - iwpriv ra0 set WscOOB=1

- Disable WPS

  - iwpriv ra0 set WscConfMode=0

- Enable WPS

  - iwpriv ra0 set WscConfMode=7 (Binary: 111)

- Set WPS SC state to un-configured

  - iwpriv ra0 set WscConfStatus=1

- Set WPS SC state to configured

  - iwpriv ra0 set WscConfStatus=2

- AP services as Enrollee by using PIN code

  - iwpriv ra0 set WscMode=1

  - iwpriv ra0 set WscGetConf=1

- AP services as Enrollee by using PBC

  - iwpriv ra0 set WscMode=2

  - iwpriv ra0 set WscGetConf=1

- AP services as Internal Registrar using PIN code

  - iwpriv ra0 set WscMode=1

  - iwpriv ra0 set WscPinCode=xxxxxxxx (PIN code from Enrollee, len=8)

  - iwpriv ra0 set WscGetConf=1

- AP services as Internal Registrar using PBC

  - iwpriv ra0 set WscMode=2

  - iwpriv ra0 set WscGetConf=1

Note.

1) AP services as Enrollee:

* If AP-Enrollee SC state is 0x1, AP will restart with new configurations.

* If AP-Enrollee SC state is 0x2, AP sends own configurations to

external-registrar and ignores configurations from external-registrar.

2) AP services as Registrar:

* If AP-Registrar SC state is 0x1, the security mode will be WPAPSK/TKIP and generate random 64bytes psk; after process, AP will restart with new security.

3) WPS AP only services one WPS client at a time.

4) WPS AP only can work in ra0.

## New files for WPS AP

wsc.c, wsc_tlv.c, sha2.c, hmac.c, dh_key.c, evp_enc.c

## New compile flag for WPS AP

WFLAGS += -DWSC_SUPPORT

## New items for RT61AP.dat file

WscConfMode=0

WscConfStatus=1

## Related Documents

WPS Specification (Simple_Config_v1.0g.pdf)

Simple Config Test Plan Draft (SCTestPlan_clean_v0_4_9.doc)